DATA

MANAGEMENT

PLAN

# Introduction

This Data Management Plan outlines how data is collected, processed, stored, and maintained to support the first iteration (Minimum Viable Product, MVP) of our pedestrian navigation system for Melbourne CBD. The MVP focuses on three core functions: displaying real-time pedestrian congestion on an interactive map, recommending routes based on historical pedestrian predictions and current crowd conditions, and providing users with a list of nearby landmarks and places of interest.

The system integrates multiple open datasets from the City of Melbourne, including real-time pedestrian counts, historical hourly pedestrian counts, sensor location information, and landmark datasets. These datasets are accessed through APIs or CSV downloads and are processed through a structured data pipeline involving extraction, cleaning, transformation, storage, and scheduled updates.

This document describes the datasets used in the project, the data flow pipeline, database design, update strategy, and data governance practices. It also outlines how the project ensures data quality, licensing compliance, privacy protection, and scalability for future iterations. The overall objective is to provide reliable and up-to-date information that supports users in choosing less crowded routes while improving their navigation experience in Melbourne CBD.

# Understanding the Problem Domain & User Needs

## Problem Domain

Melbourne CBD experiences varying levels of pedestrian congestion throughout the day, especially during commuting hours and major events. High crowd density can make navigation less efficient and reduce the overall walking experience for residents and visitors. While pedestrian sensor data is publicly available through the City of Melbourne Open Data Portal, users often lack an intuitive way to view real-time crowd conditions or use historical trends to make informed route decisions.

## User Needs

The first iteration (MVP) of the system addresses the following user needs:

View real-time pedestrian congestion on an interactive map.

Receive recommended walking routes by combining real-time crowd conditions with historical pedestrian predictions.

Discover nearby landmarks and places of interest that can be incorporated into their journey.

The system aims to help users make better navigation decisions by providing timely, data-driven insights rather than requiring them to interpret raw pedestrian datasets themselves.

# Data Source Table

| Dataset Name | Physical access | Frequency of source updates | Frequency of iteration system updates | Granularity | Copyright / licensing details | Source Link |
| --- | --- | --- | --- | --- | --- | --- |
| Pedestrian Counting System – Past Hour (Counts per Minute) | API / CSV | Real-time (updated frequently) | Every 24 hours | Minute-level counts per sensor | Creative Commons Attribution 4.0 https://creativecommons.org/licenses/by/4.0/legalcode | https://data.melbourne.vic.gov.au/explore/dataset/pedestrian-counting-system-past-hour-counts-per-minute/information/ |
| Pedestrian Counting System – Past Hour (Counts per Hour) | API / CSV | Updated after a day | Every iteration | Hour-level counts per sensor | Creative Commons Attribution 4.0 https://creativecommons.org/licenses/by/4.0/legalcode | https://data.melbourne.vic.gov.au/explore/dataset/pedestrian-counting-system-monthly-counts-per-hour/information/ |
| Pedestrian Counting System - Sensor Locations | API / CSV | Occasionally updated | Every iteration | Sensor location metadata | Creative Commons Attribution 4.0 https://creativecommons.org/licenses/by/4.0/legalcode | https://data.melbourne.vic.gov.au/explore/dataset/pedestrian-counting-system-sensor-locations/information/ |
| Landmarks and places of interest, including schools, theatres, health services, sports facilities, places of worship, galleries and museums. | API / CSV | Occasionally updated | Every iteration | description and co-ordinates of places of interest within the City of Melbourne | Creative Commons Attribution 4.0 https://creativecommons.org/licenses/by/4.0/legalcode | https://data.melbourne.vic.gov.au/explore/dataset/landmarks-and-places-of-interest-including-schools-theatres-health-services-spor/information/ |
| Additional supporting datasets (later iteration) |  |  |  |  |  |  |
| Microclimate sensors data | API / CSV | 15 minutes | Every 24 hours | Minute-level indicates per sensor | Creative Commons Attribution 4.0 https://creativecommons.org/licenses/by/4.0/legalcode | https://data.melbourne.vic.gov.au/explore/dataset/microclimate-sensors-data/information/?sort=received_at |
| On-street Parking Bay Sensors | API / CSV | 2 Minutes | Every 24 hours | Minute-level counts per sensor | Creative Commons Attribution 4.0 https://creativecommons.org/licenses/by/4.0/legalcode | https://data.melbourne.vic.gov.au/explore/dataset/on-street-parking-bay-sensors/information/ |
| Feature Lighting (including light type, wattage and location) | API / CSV | Daily | Every 24 hours | Day-level indicates per sensor | Creative Commons Attribution 4.0 https://creativecommons.org/licenses/by/4.0/legalcode | https://data.melbourne.vic.gov.au/explore/dataset/feature-lighting-including-light-type-wattage-and-location/information/ |

# Data Usage

| Feature | What the feature does | Problem it solves | Dataset used | Attributes | How the data supports the feature |
| --- | --- | --- | --- | --- | --- |
| Real-time Crowd Map | Displays current pedestrian congestion levels on an interactive map. | Users cannot easily identify crowded areas in Melbourne CBD. | Pedestrian Counting System – Past Hour (Counts per Minute); Pedestrian Counting System – Sensor Locations | Counts per Minute: Sensing DateTime, Location ID, Direction 1, Direction 2, Total of Directions Sensor Locations: Location ID, Sensor description, Latitude, Longitude | Real-time pedestrian counts are linked with sensor locations to calculate and display current crowd density on the map. |
| Crowd-aware Route Recommendation | Recommends walking routes by combining historical pedestrian patterns with current crowd conditions. | Users want to avoid congested routes and choose more comfortable paths. | Pedestrian Counting System – Past Hour (Counts per Hour); Pedestrian Counting System – Past Hour (Counts per Minute); Sensor Locations | Hourly Counts: Date, Hour, Location ID, Total of Directions Minute Counts: Location ID, Total of Directions Sensor Locations: Location ID, Latitude, Longitude | Historical hourly data is used to predict expected crowd levels, while real-time counts provide current conditions. Sensor locations enable route calculation between sensors. |
| Potential Landmarks List | Displays nearby landmarks and places of interest along or near the recommended route. | Users experiencing sensory overload may need to locate nearby potential quiet spaces | Landmarks and Places of Interest | Feature Name, Theme, Sub Theme, Co-ordinates | Landmark information is matched with the recommended route to generate a list of nearby attractions for users. |

- Data Wrangling / Data Pipeline

- Pedestrian Counting System - Sensor Locations

Input:

This pipeline uses the Sensor Locations dataset from the City of Melbourne Open Data Portal.

Raw dataset: pedestrian-counting-system-sensor-locations.csv (134 records)

Raw columns:

location_id, sensor_description, sensor_name, installation_date, note, location_type, status, direction_1, direction_2, latitude, longitude, location

Output:

sensor_locations_clean.csv (134 records × 7 columns)

Step 1. Column Filtering

Only the attributes required for the MVP are retained.

Kept:

location_id (primary key for joining datasets)

sensor_description (human-readable sensor location)

latitude (map coordinates)

longitude (map coordinates)

direction_1 (pedestrian movement direction)

direction_2 (pedestrian movement direction)

status (sensor status)

Dropped:

sensor_name (technical sensor identifier, not displayed)

installation_date (historical metadata)

note (maintenance/replacement information)

location_type (not required for MVP)

location (duplicate coordinate information)

Step 2. Data Quality Validation

The cleaned dataset was validated before export.

Validation performed:

Checked missing values

Verified location_id uniqueness (primary key)

Confirmed all retained sensors have status = "A"

Confirmed missing values in direction_1 and direction_2 only occur for indoor sensors

Step 3. Export

The cleaned dataset was exported as:

sensor_locations_clean.csv

The final dataset contains:

134 sensor locations

7 attributes

Ready for integration with the historical pedestrian counts dataset and the real-time pedestrian API.

- Pedestrian Counting System (counts per hour)

Input:

This pipeline uses the Pedestrian Counting System (counts per hour) dataset from the City of Melbourne Open Data Portal.

Raw dataset: pedestrian-counting-system-monthly-counts-per-hour.csv

Raw records: 1,613,233 rows

Raw columns:

id, location_id, sensing_date, hour_day, direction_1, direction_2, total_of_directions, sensor_name, location

Output:

historical_crowd_baseline.csv with 17,037 rows

Final attributes:

location_id, weekday_num, weekday, hour_day, average_count, median_count, d1_med, d2_med, total_med_perminute, d1_med_perminute, d2_med_perminute, sample_size

Step 1. Date filtering

The raw dataset covers the period from 4 August 2024 to 3 August 2026. To create a consistent historical baseline, only one complete year was retained:

1 August 2025 – 31 July 2026

This reduced the dataset from 1,613,233 to 825,104 records while preserving seasonal pedestrian patterns.

Step 2. Temporal feature creation

New temporal attributes were derived from sensing_date.

Created:

weekday_num: numerical weekday, where Monday = 1 and Sunday = 7

weekday: human-readable weekday name

is_weekend: temporary field used to distinguish weekday and weekend records

The final output retains weekday_num and weekday. The is_weekend field was dropped because it can be directly derived from the weekday.

Step 3. Column filtering

Only the columns required to construct the historical pedestrian baseline were retained during processing.

Kept during wrangling:

location_id — sensor identifier and join key

sensing_date — used to derive weekday information

weekday_num — backend-friendly weekday value

weekday — readable weekday label

hour_day — hour of the day

direction_1 — pedestrian count recorded in the first direction

direction_2 — pedestrian count recorded in the second direction

total_of_directions — total hourly pedestrian count

Dropped:

id — raw record identifier, not required by the application

sensor_name — sensor details are available in the separate Sensor Locations dataset

location — coordinates are available in the Sensor Locations dataset

Although direction_1 and direction_2 were retained during processing, the final baseline uses total_of_directions because the MVP requires an overall crowd level rather than separate directional counts.

Step 4. Historical aggregation

The records were grouped by:

location_id

weekday_num

weekday

hour_day

For each sensor, weekday and hour combination, the following values were calculated:

- average_count — average total pedestrian count per hour

- median_count — median total pedestrian count per hour

- d1_med — median hourly pedestrian count for Direction 1

- d2_med — median hourly pedestrian count for Direction 2

- sample_size — number of historical observations used in the calculation

The median was retained because it is less affected by unusually high pedestrian counts caused by special events, temporary crowd surges or other exceptional conditions. The mean was also included for comparison and possible backend use.

Step 5. Per-minute baseline calculation

The real-time pedestrian API provides counts at minute-level granularity. To support comparison between historical hourly data and real-time minute data, the hourly median values were divided by 60.

The following attributes were created:

total_med_perminute — total historical median count per minute

d1_med_perminute — historical median count per minute for Direction 1

d2_med_perminute — historical median count per minute for Direction 2

Step 6. Validation

The processed dataset was validated by checking:

- missing values;

- duplicate combinations of location_id, weekday_num and hour_day;

- sample-size distribution;

- completeness of expected weekday-hour combinations;

- minimum and maximum pedestrian count values.

Most sensors contain the expected:

7 weekdays × 24 hours

Some sensors contain fewer records because of gaps in the original hourly observations. These records were retained, and sample_size was included to show how much historical data supports each calculated baseline.

Step 7. Export

The final processed dataset was exported as:

historical_crowd_baseline.csv

Attribute		Purpose

location_id		Connects the baseline to the sensor location and live API datasets

weekday_num		Numerical weekday for backend queries

weekday		Human-readable weekday label

hour_day		Hour of the day from 0 to 23

average_count	Mean historical total pedestrian count per hour

median_count		Typical historical total pedestrian count per hour

d1_med		Median hourly count for Direction 1

d2_med		Median hourly count for Direction 2

total_med_perminute	Approximate total historical median count per minute

d1_med_perminute	Approximate Direction 1 historical median count per minute

d2_med_perminute	Approximate Direction 2 historical median count per minute

sample_size		Number of historical observations supporting each result

The final dataset contains 17,037 rows and supports:

- comparison between real-time and historical pedestrian levels;

- crowd-density classification;

- heatmap generation;

- direction-aware sensory-friendly route recommendations.

  - Explain how raw data is transformed into clean/client-ready data.

  - For each pipeline, include:

    - Pipeline name

    - Input dataset ID and file name

    - Input format, e.g. CSV, JSON, API response

    - Key columns in the raw data

    - Step-by-step processing actions

    - Columns kept

    - Columns removed

    - Missing value handling

    - Data type conversion

    - Filtering rules

    - Transformation logic

    - Output file name

    - Output format

    - Expected number of rows/columns if known

  - Avoid vague descriptions like “clean the data” or “drop some columns”.

- Data Storage

  - Explain how cleaned data is stored in the system.

  - Since relational database is mandatory, include:

    - Database type, e.g. MySQL, PostgreSQL

    - Hosting/deployment platform, e.g. AWS RDS, EC2

    - Database name

    - Tables created

    - Mapping between clean files and database tables

    - Import method, e.g. SQL script, Python script, GUI, migration tool

  - If using other storage, e.g. S3 bucket, explain what is stored there and how the system accesses it.

  - Note: Firebase requires strong technical justification and mentor approval; Supabase is not allowed according to the video.

- Entity Relationship Diagram / ERD

  - Include a clear screenshot of the ERD.

  - Explain:

    - Main tables

    - Primary keys

    - Foreign keys

    - Relationships between tables

    - How the database structure supports the system features

  - This section acts as the blueprint for rebuilding the database.

- Data Analysis

  - Explain how analysis is connected to user requirements and product goals.

  - Include:

    - User requirement being addressed

    - Analysis method used

    - Tools used

    - Preliminary experiments

    - Key findings

  - You can structure findings as:

    - Hindsight: what happened in the data

    - Insight: what patterns or relationships were found

    - Foresight: how findings support prediction, recommendation, planning, or decision making

- Data Backup and Archive

  - Explain how the team backs up:

    - Raw datasets

    - Clean datasets

    - Database exports

    - Scripts

  - Include:

    - Backup location, e.g. GitHub, shared drive, cloud folder

    - Folder structure

    - File naming convention

    - Backup frequency

    - When backups are created, e.g. end of each iteration

    - How old versions are archived

  - Make sure the backup plan is realistic and actually followed.

- Ethical, Legal and Privacy Issues

  - Identify risks related to:

    - Bias

    - Unfair recommendations or predictions

    - Misleading outputs

    - Copyright and licensing

    - Attribution requirements

    - Terms of use

    - Personal or sensitive information

    - Location data

    - Re-identification risks

  - Also explain how the team manages these issues, such as removing sensitive fields, checking licenses, explaining model limitations, or avoiding unsupported claims.

- Appendix / Other Work

  - Include relevant extra technical work that does not fit in the main sections.

  - Examples:

    - Machine learning model training

    - Deep learning experiments

    - Feature selection

    - Model evaluation

    - Deployment details

    - Additional screenshots

    - Extra scripts or diagrams

  - Only include work that supports the product, data pipeline, or data-driven features.